Jarvis Viper Slide Test

Java, Encoders, Presets, Telemetry, and Debugging

A detailed FTC Java guide written by Artemis Robotics

For beginner programmers: learn what every important line does, how the Viper slide moves, and how engineers debug a robot when it does not behave as expected.

| Part | What students will learn |
| --- | --- |
| Java basics | Classes, objects, methods, variables, booleans, if/else, loops, enums, return values. |
| FTC basics | OpModes, hardwareMap, gamepad1, telemetry, motor run modes. |
| Viper mechanics | Encoder ticks, target position, motor power, presets, holding position. |
| Control flow | Manual control vs preset control and how the program chooses between them. |
| Debugging | How we isolated gamepad, motor power, encoder direction, and preset-state problems. |

# 1. What Are We Testing?

This OpMode is intentionally small. It tests only the Viper slide. It does not create the full HyperionRobot, so it does not need the drivetrain, intake, Limelight, or Pinpoint odometry.

Driver presses a Viper control

↓

JarvisViper reads gamepad1

↓

JarvisViper calls a ViperArm method

↓

ViperArm sets a motor target

↓

Motor controller drives the slide

↓

Encoder reports the new position

↓

Telemetry shows what happened

↓

## Why isolate one subsystem?

When only one subsystem is being tested, fewer things can go wrong. If the Viper test fails, we know the problem is likely in the gamepad, Viper code, motor wiring, encoder, or slide mechanism—not the camera or drivetrain.

# 2. The Java Pieces You Need to Know First

| Java idea | Simple explanation | Example from this project |
| --- | --- | --- |
| Class | A blueprint for an object. | ViperArm, JarvisViper |
| Object | A real instance made from a class. | viper |
| Method | An action a class/object can perform. | viper.jog(1.0) |
| Variable | A named place that stores a value. | presetActive |
| boolean | A value that is only true or false. | manualControlActive |
| if / else | Choose which code runs based on a condition. | If L1 is pressed, extend. |
| while loop | Repeat code while a condition is true. | while (opModeIsActive()) |
| enum | A fixed set of named choices. | STOWED, LOW, MID, HIGH, MAX |
| return | Send a value back, or exit a method. | return; / getCurrentTicks() |
| null | No object is available. | secondary is null for a single-motor slide. |

# 3. Complete JarvisViper Test Code

This teaching version includes the preset-state fix: a single press selects a preset, and the slide continues moving toward that encoder target after the button is released.

package org.firstinspires.ftc.teamcode.HyperionRobotics.opmodes;

import com.qualcomm.robotcore.eventloop.opmode.LinearOpMode;
import com.qualcomm.robotcore.eventloop.opmode.TeleOp;

import org.firstinspires.ftc.teamcode.HyperionRobotics.viper.ViperArm;

/**
 * Viper Slide ONLY Test
 *
 * No HyperionRobot, drivetrain, intake, Pinpoint, or Limelight.
 *
 * Controls:
 *   Left Bumper  = Extend manually
 *   Left Trigger = Retract manually
 *
 *   D-Pad Up     = LOW
 *   D-Pad Right  = MID
 *   D-Pad Down   = HIGH
 *   D-Pad Left   = MAX
 *   B / Circle   = STOWED
 */
@TeleOp(name = "Jarvis Viper Test", group = "Test")
public class JarvisViper extends LinearOpMode {

    private ViperArm viper;

    /*
     * true means a preset was selected and RUN_TO_POSITION should
     * continue toward that preset even after the button is released.
     */
    private boolean presetActive = false;

    @Override
    public void runOpMode() {

        // Initialize ONLY the Viper subsystem.
        viper = new ViperArm(hardwareMap);

        telemetry.setMsTransmissionInterval(50);

        telemetry.addLine("================================");
        telemetry.addLine("       VIPER SLIDE TEST");
        telemetry.addLine("================================");
        telemetry.addLine("Left Bumper  = MANUAL UP");
        telemetry.addLine("Left Trigger = MANUAL DOWN");
        telemetry.addLine("D-Pad Up     = LOW");
        telemetry.addLine("D-Pad Right  = MID");
        telemetry.addLine("D-Pad Down   = HIGH");
        telemetry.addLine("D-Pad Left   = MAX");
        telemetry.addLine("B / Circle   = STOWED");
        telemetry.addLine("Waiting for START...");
        telemetry.update();

        waitForStart();

        if (isStopRequested()) {
            viper.stop();
            return;
        }

        while (opModeIsActive()) {

            boolean manualControlActive = false;
            boolean presetRequestedThisLoop = false;
            String command = "HOLD";

            // ============================================================
            // 1. MANUAL CONTROL
            // ============================================================

            if (gamepad1.left_bumper) {

                // Manual command takes control away from any preset.
                presetActive = false;
                manualControlActive = true;
                command = "MANUAL UP";

                viper.jog(1.0);

            } else if (gamepad1.left_trigger > 0.1) {

                presetActive = false;
                manualControlActive = true;
                command = "MANUAL DOWN";

                viper.jog(-1.0);
            }

            // ============================================================
            // 2. PRESET CONTROL
            // ============================================================

            /*
             * Only allow a preset command when manual control is not active.
             * Once selected, presetActive remains true after the button is
             * released so RUN_TO_POSITION can finish the move.
             */
            if (!manualControlActive) {

                if (gamepad1.dpad_up) {

                    viper.setStage(ViperArm.Stage.LOW);
                    presetActive = true;
                    presetRequestedThisLoop = true;
                    command = "PRESET LOW";

                } else if (gamepad1.dpad_right) {

                    viper.setStage(ViperArm.Stage.MID);
                    presetActive = true;
                    presetRequestedThisLoop = true;
                    command = "PRESET MID";

                } else if (gamepad1.dpad_down) {

                    viper.setStage(ViperArm.Stage.HIGH);
                    presetActive = true;
                    presetRequestedThisLoop = true;
                    command = "PRESET HIGH";

                } else if (gamepad1.dpad_left) {

                    viper.setStage(ViperArm.Stage.MAX);
                    presetActive = true;
                    presetRequestedThisLoop = true;
                    command = "PRESET MAX";

                } else if (gamepad1.circle || gamepad1.b) {

                    viper.setStage(ViperArm.Stage.STOWED);
                    presetActive = true;
                    presetRequestedThisLoop = true;
                    command = "PRESET STOWED";
                }
            }

            // ============================================================
            // 3. HOLD LOGIC
            // ============================================================

            /*
             * IMPORTANT:
             * Do NOT call holdPosition() while a preset is active.
             *
             * RUN_TO_POSITION already keeps moving toward the stage target.
             * Calling holdPosition() every loop would replace that target
             * with the current position and stop the preset early.
             */
            if (!manualControlActive && !presetActive) {

                viper.holdPosition();
                command = "HOLD";

            } else if (presetActive && !presetRequestedThisLoop) {

                command = viper.isAtTarget()
                        ? "PRESET REACHED / HOLDING"
                        : "MOVING TO PRESET";
            }

            // ============================================================
            // 4. TELEMETRY
            // ============================================================

            int currentTicks = viper.getCurrentTicks();
            int targetTicks = viper.getTargetTicks();
            int positionError = viper.getPositionError();

            telemetry.addLine("========== VIPER SLIDE ==========");
            telemetry.addData("Command", command);
            telemetry.addData("Current Position", "%d ticks", currentTicks);
            telemetry.addData("Target Position", "%d ticks", targetTicks);
            telemetry.addData("Position Error", "%d ticks", positionError);
            telemetry.addData("At Target", viper.isAtTarget() ? "YES" : "NO");
            telemetry.addData("Selected Stage", viper.getCurrentStage());
            telemetry.addData("Stage Target", "%d ticks",
                    viper.getCurrentStage().ticks);
            telemetry.addData("Preset Active", presetActive);
            telemetry.addData("Manual Active", manualControlActive);

            telemetry.addLine();
            telemetry.addLine("---------- GAMEPAD ----------");
            telemetry.addData("L1 / Left Bumper", gamepad1.left_bumper);
            telemetry.addData("L2 / Left Trigger", "%.2f",
                    gamepad1.left_trigger);
            telemetry.addData("D-Pad Up", gamepad1.dpad_up);
            telemetry.addData("D-Pad Right", gamepad1.dpad_right);
            telemetry.addData("D-Pad Down", gamepad1.dpad_down);
            telemetry.addData("D-Pad Left", gamepad1.dpad_left);
            telemetry.addData("B / Circle",
                    gamepad1.b || gamepad1.circle);

            telemetry.update();
            idle();
        }

        viper.stop();
    }
}

# 4. How the OpMode Starts

@TeleOp(name = "Jarvis Viper Test", group = "Test") — Registers this Java class as a Driver Station TeleOp OpMode.

Java lesson: An annotation begins with @ and adds information about a class.

In the Viper test: It gives the program the name you select on the Driver Station.

public class JarvisViper extends LinearOpMode — Defines the JarvisViper class and says it is an FTC LinearOpMode.

Java lesson: "extends" means JarvisViper inherits useful behavior from LinearOpMode.

In the Viper test: This gives us waitForStart(), opModeIsActive(), gamepad1, telemetry, and hardwareMap.

private ViperArm viper; — Declares a variable that will refer to our ViperArm object.

Java lesson: "private" means the variable is intended for use inside this class. ViperArm is the type; viper is the variable name.

In the Viper test: This replaces the old HyperionRobot dependency.

private boolean presetActive = false; — Remembers whether a preset move is currently active.

Java lesson: boolean variables hold only true or false. They are excellent for remembering program state.

In the Viper test: This fixes the problem where releasing the D-pad accidentally stopped a preset move.

public void runOpMode() — The main method FTC calls for this LinearOpMode.

Java lesson: "void" means this method does not return a value.

In the Viper test: Everything from initialization to the main control loop starts here.

# 5. Connecting Java to the Viper Hardware

viper = new ViperArm(hardwareMap); — Creates the ViperArm subsystem and gives it access to configured robot hardware.

Java lesson: "new" constructs a new object. The value inside parentheses is an argument passed to the constructor.

In the Viper test: ViperArm uses hardwareMap to find the motor named by RobotConstants.VIPER_MOTOR.

hardwareMap.get(DcMotorEx.class, name) — Finds a configured motor by name.

Java lesson: A method can accept multiple arguments. Here one says what type of hardware we want, and one says its configured name.

In the Viper test: Inside ViperArm.safeGet(), this is how the code finds viperMotor.

## Why the old HyperionRobot caused an odometry requirement

The earlier test created new HyperionRobot(hardwareMap). That full robot object initialized other subsystems, including Pinpoint. The Viper-only test creates only ViperArm, so unrelated hardware cannot block the test.

# 6. Motor Initialization and Encoder Setup

motor.setZeroPowerBehavior(DcMotor.ZeroPowerBehavior.BRAKE);
motor.setMode(DcMotor.RunMode.STOP_AND_RESET_ENCODER);
motor.setTargetPosition(0);
motor.setMode(DcMotor.RunMode.RUN_TO_POSITION);
motor.setPower(0.0);

setZeroPowerBehavior(BRAKE) — Tells the motor controller to resist free-spinning when commanded power is zero.

Java lesson: The value inside parentheses is an enum constant.

In the Viper test: Useful for a vertical slide because we usually do not want it to coast freely.

setMode(STOP_AND_RESET_ENCODER) — Sets the motor's encoder count to zero.

Java lesson: A method changes the internal state of an object.

In the Viper test: The code assumes the slide starts at its known STOWED position when zeroed.

Important: Only reset at a known physical reference. Resetting at an arbitrary position makes all presets wrong.

setTargetPosition(0) — Sets the first encoder destination to zero ticks.

In the Viper test: RUN_TO_POSITION needs a target.

setMode(RUN_TO_POSITION) — Turns on closed-loop position control using the encoder.

Java lesson: "Closed-loop" means the controller compares where it is with where it should be.

In the Viper test: The motor keeps trying to reach the requested tick target.

Important: Do not unplug the encoder while RUN_TO_POSITION is commanding a distant target.

setPower(RobotConstants.VIPER_POWER) — Sets how strongly the motor controller is allowed to drive toward its target.

Java lesson: The constant is a double such as 0.90.

In the Viper test: Preset moves use this power while RUN_TO_POSITION decides when to stop.

# 7. Motor Direction and Why Positive Encoder Ticks Matter

During debugging, extending the slide originally produced negative encoder values. The stage model expects extension to be positive:

STOWED = 0
LOW    = 900
MID    = 1800
HIGH   = 2700
MAX    = 3600

The direction was reversed so the working convention became: extend -> positive ticks, retract -> ticks move back toward zero.

primary.setDirection(DcMotor.Direction.REVERSE) — Reverses the motor's logical direction convention.

Java lesson: Direction.REVERSE is an enum value.

In the Viper test: On this robot it makes the slide's extension agree with positive encoder targets.

# 8. Waiting for the Driver and Repeating the Control Loop

waitForStart() — Pauses until the Driver Station operator presses START.

In the Viper test: Hardware is initialized before START, but the slide does not begin normal control until START.

isStopRequested() — Checks whether the OpMode has already been asked to stop.

Java lesson: A method that returns true/false can be used directly in an if condition.

In the Viper test: If true, the code stops the Viper and exits.

while (opModeIsActive()) — Repeats the control code while the OpMode is running.

Java lesson: A while loop repeats its block as long as the condition remains true.

In the Viper test: This is why the robot continuously reads the gamepad and updates telemetry.

idle() — Briefly yields control to the FTC runtime.

In the Viper test: It is a polite way for a LinearOpMode loop to avoid hogging processor time.

# 9. Manual Control Flow

Check gamepad1.left_bumper

↓

If true: cancel presetActive

↓

Call viper.jog(+1.0)

↓

ViperArm reads current encoder position

↓

Add about +40 ticks

↓

Call goToTicks(new target)

↓

RUN_TO_POSITION moves toward that nearby target

↓

gamepad1.left_bumper — A boolean that is true while L1 / Left Bumper is held.

Java lesson: Objects can contain fields. gamepad1 is an object; left_bumper is one of its fields.

In the Viper test: L1 extends the slide.

gamepad1.left_trigger — A number representing how far L2 / Left Trigger is pressed.

Java lesson: Unlike a bumper, a trigger is analog, so it is a floating-point value rather than only true/false.

In the Viper test: The code checks > 0.1 so tiny trigger noise is ignored.

viper.jog(1.0) / viper.jog(-1.0) — Moves the manual target a small number of encoder ticks in one direction.

Java lesson: A positive or negative double is passed as an argument.

In the Viper test: This is NOT direct open-loop motor power. It repeatedly creates nearby RUN_TO_POSITION targets.

## Why manual jog can feel slower than VIPER_POWER suggests

VIPER_POWER may be 0.90, but jog() advances the target by only about 40 ticks per loop:

int next = primary.getCurrentPosition() + (int)(power * 40);

So VIPER_POWER controls how strongly the motor tries to reach each small target, while the 40-tick step limits how quickly the requested target moves. That is why direct RUN_WITHOUT_ENCODER testing at 0.50 could feel faster.

# 10. Preset Control: One Press, Then Continue Automatically

Driver presses one D-pad direction

↓

Jarvis calls viper.setStage(...)

↓

presetActive = true

↓

Driver releases button

↓

Jarvis does NOT call holdPosition()

↓

RUN_TO_POSITION keeps moving toward the preset

↓

isAtTarget() becomes true near the target

↓

Motor controller holds the target

↓

viper.setStage(ViperArm.Stage.HIGH) — Selects a named stage and sends its tick target to the motor.

Java lesson: Stage.HIGH is an enum constant. Enums make code easier to read than using raw numbers everywhere.

In the Viper test: HIGH maps to RobotConstants.VIPER_STAGE_3.

presetActive = true — Remembers that the robot should continue a preset move after the button is released.

Java lesson: Assignment uses =. Comparison usually uses ==. This line changes the variable's stored value.

In the Viper test: This is the key state-memory fix.

viper.isAtTarget() — Checks whether current position is close enough to target position.

Java lesson: This method returns a boolean.

In the Viper test: Telemetry can show when the preset has reached its tolerance window.

Math.abs(currentPosition - targetPosition)
        <= RobotConstants.VIPER_TOLERANCE

Math.abs() removes the sign so an error of -18 ticks and +18 ticks are treated as the same size error.

# 11. holdPosition(): Useful, but Easy to Misuse

viper.holdPosition() — Changes the target to the slide's current encoder position.

In the Viper test: Useful after manual control stops, because it tells the motor to stay where the driver released it.

Important: Do not call it every loop while a preset is still traveling.

## The preset bug we found

// Problem behavior:
Press MID -> target = 1800

Release D-pad
    ↓
holdPosition()
    ↓
target = current position, maybe 420

Result: slide stops around 420 instead of continuing to 1800

The fix is to call holdPosition() only when neither manual control nor an active preset owns the slide.

# 12. What Happens Inside ViperArm.setStage()

setStage(Stage.MID)

↓

currentStage = MID

↓

MID.ticks is read

↓

goToTicks(1800)

↓

Range.clip() checks limits

↓

setTargetPosition(1800)

↓

RUN_TO_POSITION

↓

setPower(VIPER_POWER)

↓

currentStage = stage — Stores the most recently requested named stage.

Java lesson: The parameter stage is a local variable passed into setStage().

In the Viper test: Telemetry can report the selected stage.

stage.ticks — Reads the integer tick value stored inside that enum value.

Java lesson: Enums can contain data and even constructors, not just names.

In the Viper test: LOW, MID, HIGH, and MAX each carry their own encoder target.

Range.clip(ticks, 0, VIPER_MAX_TICKS) — Forces a requested target to stay inside allowed limits.

In the Viper test: A request below 0 becomes 0; a request above the maximum becomes the maximum.

primary.setTargetPosition(clipped) — Stores the encoder destination in the motor controller.

In the Viper test: This is where a preset becomes a real motor target.

# 13. Telemetry: The Robot Explaining Its Thinking

telemetry.addData(label, value) — Adds one labeled value to the Driver Station display.

Java lesson: Methods can be overloaded, meaning the same method name can accept different kinds of arguments.

In the Viper test: Shows command, positions, error, stage, buttons, and state.

telemetry.update() — Sends the prepared telemetry to the Driver Station.

In the Viper test: Without update(), new telemetry may not appear immediately.

| Telemetry | What it tells us |
| --- | --- |
| Command | Which control decision Jarvis made this loop. |
| Current Position | Where the encoder says the slide is now. |
| Target Position | Where RUN_TO_POSITION is trying to go. |
| Position Error | Target minus current. |
| At Target | Whether error is inside the allowed tolerance. |
| Selected Stage | Last named preset selected. |
| Preset Active | Whether a preset still owns the slide. |
| Manual Active | Whether L1/L2 is controlling it now. |
| Gamepad fields | Whether Driver Station input is actually reaching the code. |

# 14. Relevant ViperArm Code

JarvisViper is the driver/controller layer. ViperArm is the subsystem layer that talks to the motor and encoder. These are the methods JarvisViper depends on.

public class ViperArm {

    public enum Stage {
        STOWED(RobotConstants.VIPER_STAGE_0),
        LOW(RobotConstants.VIPER_STAGE_1),
        MID(RobotConstants.VIPER_STAGE_2),
        HIGH(RobotConstants.VIPER_STAGE_3),
        MAX(RobotConstants.VIPER_STAGE_4);

        public final int ticks;

        Stage(int ticks) {
            this.ticks = ticks;
        }
    }

    private final DcMotorEx primary;
    private final DcMotorEx secondary;
    private Stage currentStage = Stage.STOWED;

    public ViperArm(HardwareMap hardwareMap) {
        primary = safeGet(hardwareMap, RobotConstants.VIPER_MOTOR);

        if (RobotConstants.VIPER_DUAL_MOTOR) {
            secondary = safeGet(hardwareMap, RobotConstants.VIPER_MOTOR_2);
        } else {
            secondary = null;
        }

        if (primary != null) {
            configureMotor(primary);

            // On this robot, reversing makes extension count positive.
            primary.setDirection(DcMotor.Direction.REVERSE);
        }

        if (secondary != null) {
            configureMotor(secondary);
            secondary.setDirection(DcMotor.Direction.REVERSE);
        }

        if (primary != null) {
            setStage(Stage.STOWED);
        }
    }

    private void configureMotor(DcMotorEx motor) {
        motor.setZeroPowerBehavior(DcMotor.ZeroPowerBehavior.BRAKE);
        motor.setMode(DcMotor.RunMode.STOP_AND_RESET_ENCODER);
        motor.setTargetPosition(0);
        motor.setMode(DcMotor.RunMode.RUN_TO_POSITION);
        motor.setPower(0.0);
    }

    public void setStage(Stage stage) {
        currentStage = stage;
        goToTicks(stage.ticks);
    }

    public void goToTicks(int ticks) {
        if (primary == null) return;

        int clipped = Range.clip(
                ticks,
                0,
                RobotConstants.VIPER_MAX_TICKS
        );

        primary.setTargetPosition(clipped);
        primary.setMode(DcMotor.RunMode.RUN_TO_POSITION);
        primary.setPower(RobotConstants.VIPER_POWER);

        if (secondary != null) {
            secondary.setTargetPosition(clipped);
            secondary.setMode(DcMotor.RunMode.RUN_TO_POSITION);
            secondary.setPower(RobotConstants.VIPER_POWER);
        }
    }

    public void jog(double power) {
        if (primary == null) return;

        if (Math.abs(power) < 0.05) {
            holdPosition();
            return;
        }

        int next = Range.clip(
                primary.getCurrentPosition() + (int)(power * 40),
                0,
                RobotConstants.VIPER_MAX_TICKS
        );

        goToTicks(next);
    }

    public void holdPosition() {
        if (primary == null) return;
        goToTicks(primary.getCurrentPosition());
    }

    public void stop() {
        if (primary != null) primary.setPower(0.0);
        if (secondary != null) secondary.setPower(0.0);
    }

    public boolean isAtTarget() {
        if (primary == null) return true;

        return Math.abs(
                primary.getCurrentPosition()
                - primary.getTargetPosition()
        ) <= RobotConstants.VIPER_TOLERANCE;
    }

    public int getCurrentTicks() {
        return (primary != null)
                ? primary.getCurrentPosition()
                : 0;
    }

    public int getTargetTicks() {
        return (primary != null)
                ? primary.getTargetPosition()
                : 0;
    }

    public int getPositionError() {
        return getTargetTicks() - getCurrentTicks();
    }

    public Stage getCurrentStage() {
        return currentStage;
    }
}

# 15. Complete Method Call Map

| Call | Where it comes from | Purpose |
| --- | --- | --- |
| new ViperArm(hardwareMap) | Our code | Build only the Viper subsystem. |
| hardwareMap.get(...) | FTC SDK | Find the configured motor. |
| setDirection(REVERSE) | FTC motor API | Make extension agree with positive ticks. |
| setZeroPowerBehavior(BRAKE) | FTC motor API | Resist coasting at zero power. |
| STOP_AND_RESET_ENCODER | FTC motor mode | Define current known position as zero. |
| RUN_TO_POSITION | FTC motor mode | Use encoder feedback to reach a target. |
| setTargetPosition() | FTC motor API | Set destination ticks. |
| setPower() | FTC motor API | Set allowed drive effort. |
| waitForStart() | LinearOpMode | Wait for Driver Station START. |
| opModeIsActive() | LinearOpMode | Keep looping only while active. |
| gamepad1.left_bumper | FTC Gamepad | Read L1. |
| gamepad1.left_trigger | FTC Gamepad | Read L2 analog value. |
| setStage() | ViperArm | Select a named preset. |
| goToTicks() | ViperArm | Command a raw encoder target. |
| jog() | ViperArm | Move manual target by a small increment. |
| holdPosition() | ViperArm | Change target to current position. |
| isAtTarget() | ViperArm | Test position error against tolerance. |
| getCurrentTicks() | ViperArm | Read encoder position. |
| getTargetTicks() | ViperArm | Read motor target position. |
| getPositionError() | ViperArm | Compute target-current. |
| getCurrentStage() | ViperArm | Read last selected preset enum. |
| telemetry.addData() | FTC Telemetry | Prepare debugging information. |
| telemetry.update() | FTC Telemetry | Send it to Driver Station. |
| idle() | LinearOpMode | Yield briefly to FTC runtime. |
| stop() | ViperArm | Set Viper motor power to zero. |

# 16. Engineering Debugging Story: What Went Wrong and How We Found It

Debugging means finding the smallest test that separates possible causes. We did not solve the Viper by guessing one giant fix. We removed dependencies and tested one layer at a time.

| Step | Symptom | Root cause / evidence | Resolution |
| --- | --- | --- | --- |
| 1 | Viper test unexpectedly required odometry. | JarvisViper created HyperionRobot, which initialized Pinpoint and other subsystems. | Create ViperArm directly. Viper-only OpMode no longer depends on odometry. |
| 2 | Buttons appeared to do nothing. | Telemetry showed gamepad1 buttons always false/zero. Controller had been associated as Gamepad 2. | Verify Driver Station gamepad assignment and read both gamepad1/gamepad2 during diagnosis. |
| 3 | Preset showed target but motor only twitched. | Motor power cable was disconnected; encoder still produced some position information. | Reconnect motor power. Separate motor power path from encoder feedback path. |
| 4 | Unplugging encoder caused slide to run continuously. | RUN_TO_POSITION still had a far target but no valid feedback telling it that the target had been reached. | Never disconnect encoder during an active RUN_TO_POSITION test. Pulling power was the correct emergency response. |
| 5 | Needed to know whether motor or encoder was bad. | Full ViperArm mixed position control and motor control together. | Create ViperMotorEncoderDebug using RUN_WITHOUT_ENCODER, direct low motor power, and encoder telemetry. |
| 6 | At 0.15 test power the motor barely moved. | The loaded slide required more power to overcome friction/load. | Increase debug power to 0.50 for short controlled tests; slide then moved up/down. |
| 7 | Manual extension showed negative encoder ticks. | Motor/encoder direction convention was opposite the preset model. | Reverse motor direction in the active test code. Confirm extension produces positive ticks. |
| 8 | Changing ViperArm direction did not change debug results. | Debug OpMode did not use ViperArm at all. | Apply changes in the code actually being executed. This is a key debugging lesson. |
| 9 | Presets moved only while D-pad was held. | After release, holdPosition() replaced the preset target with the current position. | Add presetActive state; do not call holdPosition() while a preset is traveling. |
| 10 | Manual jog felt slower than expected even with VIPER_POWER=0.90. | jog() advances target by only ~40 ticks each loop; it is incremental position control, not direct 90% power. | Understand the difference between motor power and how quickly the target itself is moved. |

# 17. The Debugging Method We Used

Observe the physical behavior

↓

Read telemetry

↓

Decide which layer could cause it

↓

Remove unrelated subsystems

↓

Create the smallest possible test

↓

Change one thing at a time

↓

Verify with numbers, not guesses

↓

Only then return to full code

↓

## A powerful example

Motor did not move.
Encoder changed when slide moved by hand.

Conclusion:
- Encoder path was alive.
- Motor-power path still needed investigation.

Then we found:
- Motor power cable was disconnected.

The important lesson is that one working signal does not prove the whole subsystem is wired correctly. A motor has a power path and an encoder feedback path; they can fail independently.

# 18. Safety Rules for Encoder-Controlled Slides

| Rule | Why |
| --- | --- |
| Keep the encoder connected during RUN_TO_POSITION. | The controller needs feedback to know when to stop. |
| Start presets from a known zero position. | All stage targets are measured from zero. |
| Test small targets before MAX. | A direction or wiring mistake is less dangerous at 200 ticks than at 3600. |
| Keep STOP/power access close during first tests. | A runaway slide can hit a hard stop quickly. |
| Do not reset the encoder at an arbitrary physical position. | That makes the software's coordinate system wrong. |
| Use telemetry during every new mechanism test. | Current/target/error values reveal what the code believes. |

# 19. Beginner Java Exercises for the Team

1. Find three boolean variables or boolean expressions in JarvisViper.

2. What is the difference between = and == in Java?

3. Why is left_trigger a number but left_bumper is a boolean?

4. What does presetActive remember between loop iterations?

5. What would happen if presetActive were a local variable set to false at the top of every loop?

6. Why does manual control set presetActive = false?

7. What is an enum? Name all five ViperArm.Stage values.

8. What value does isAtTarget() return?

9. What does Range.clip() protect us from?

10. Why is getCurrentStage() not necessarily the same as the slide's exact physical position after manual jogging?

11. Why can VIPER_POWER be 0.90 while manual jog still feels slow?

12. Explain in your own words why unplugging the encoder during RUN_TO_POSITION is dangerous.

# 20. Glossary

| Term | Meaning |
| --- | --- |
| Encoder | A sensor that reports motor rotation as counts/ticks. |
| Tick | One unit of encoder position. |
| Target position | The encoder count the controller is trying to reach. |
| Position error | Target position minus current position. |
| Tolerance | How close is close enough to call the target reached. |
| Closed-loop control | Control that uses sensor feedback to correct what the motor does. |
| Open-loop control | Control that applies a command without using feedback to reach a specific target. |
| RUN_TO_POSITION | FTC motor mode that uses encoder feedback to reach target ticks. |
| RUN_WITHOUT_ENCODER | FTC motor mode where power is commanded directly; encoder may still be read. |
| Preset | A saved target position such as LOW, MID, or HIGH. |
| State | Information the program remembers about what it is doing now. |
| State machine | Program logic organized into modes/states and rules for changing between them. |
| Enum | A Java type containing a fixed set of named choices. |
| Constructor | Special method used when creating a new object. |
| Argument | A value passed into a method or constructor. |
| Parameter | The variable inside a method that receives an argument. |
| Boolean | A true/false Java data type. |
| double | A Java numeric type that can store decimal values. |
| int | A Java numeric type for whole numbers. |
| Cast | Explicitly convert one Java type to another, such as (int)(power * 40). |
| Telemetry | Information the robot sends to the Driver Station for people to read. |
| Subsystem | One functional part of the robot, such as ViperArm or drivetrain. |